PARKINSON’S DISEASE TREMOR DETECTION AND CLASSIFICATION USING MACHINE LEARNING MODEL IN STM 32 CUBE IDE

Guide to Run:

# 1) What you need (hardware & software)

Hardware:

- STM32 Nucleo-F401RE board (or a board with STM32F401RE).

- MPU-9255 sensor module (SPI pins broken out).

- USB cable (for ST-Link on the Nucleo).

- Jumper wires and breadboard.

- (Optional) PC with 3.3V logic only — Nucleo provides 3.3V, so OK.

Software:

- STM32CubeIDE installed (download from ST and install).

- A terminal program built into CubeIDE (Serial Terminal) or external like PuTTY / Tera Term.

# 2) Wiring (very important)

Do this BEFORE powering the board.

MPU-9255 pins → Nucleo-F401RE pins:

- VCC → 3V3 (Nucleo 3.3V)

- GND → GND

- SCLK → PA5 (SPI1_SCK)

- MOSI → PA7 (SPI1_MOSI)

- MISO → PA6 (SPI1_MISO)

- CS → PA4 (we will control this GPIO manually)

- INT → PA0 (optional — used only if you want interrupt-driven data ready)

- FSYNC → leave disconnected

Make sure ground of MPU and Nucleo are common (connected). Pull CS high by default (we will set this in code).

# 3) Open STM32CubeIDE and create a new project

- Launch STM32CubeIDE.

- File → New → STM32 Project.

- In “Board/Part Number” search box type F401RE and pick NUCLEO-F401RE (or the correct part).

- Click Next → give project name TremorDetector → Finish.

Cube configuration tool will open.

# 4) Configure Peripherals (clickable steps)

In the device configuration tree on the left:

A. SPI1

- Click Connectivity → SPI1.

- Mode: Full-Duplex Master.

- Keep NSS disabled (we use PA4 as software CS).

- Make sure the assigned pins are PA5 (SCK), PA6 (MISO), PA7 (MOSI).

B. GPIO

- Click GPIO → click pin PA4 on the pin diagram → Mode: GPIO_Output → Label it MPU_CS.

- (Optional) Click PA0 → Mode: GPIO_EXTI0 → Label MPU_INT if you want interrupts.

C. USART2 (debug print)

- Connectivity → USART2 → Mode: Asynchronous.

- TX: PA2, RX: PA3.

- Baudrate: 115200 (default).

D. System Clock

- Click Clock Configuration.

- If using the Nucleo default, set SYSCLK to 84 MHz (CubeIDE often does this automatically). Leave defaults if unsure.

E. Project Settings

- Click Project → Generate Code (or the Generate Code button).

- Choose STM32CubeIDE project type if prompted.

CubeIDE will generate initial code and open the project.

# 5) Add driver files (mpu9255.c / mpu9255.h)

In Project Explorer:

- Right-click Core/Inc → New → File → name mpu9255.h. Paste the header code.

- Right-click Core/Src → New → File → name mpu9255.c. Paste the C file code.

# 6) Edit main.c with tremor detection code

Open Core/Src/main.c generated by CubeIDE. Replace the main loop area with the code given (or carefully merge it). The key is to call mpu9255_init() and then perform sampling.

Important: Keep the CubeMX-generated SystemClock_Config(), MX_GPIO_Init(), MX_SPI1_Init(), MX_USART2_UART_Init() functions — they’re in the generated files and must remain.

# 7) Build & flash the firmware

- Click the hammer icon (Build) — fix any errors that show up. Typical fixes:

  - If math.h M_PI undefined, add #define _USE_MATH_DEFINES or use 3.14159f.

  - Make sure mpu9255.h is included and compiled (check in Project Explorer → Right-click → Refresh).

- Connect Nucleo to PC via USB.

- Click the green Run (bug → Debug) or use Run → Run As → STM32 MCU Application.

- CubeIDE will flash and start the program.

# 8) Open Serial Monitor to see output

- In CubeIDE bottom panel, open Terminal or Serial Monitor.

- Select the ST-LINK VCOM port (CubeIDE usually lists it).

- Baud = 115200, 8, N, 1.

- You should see lines like:

=== Tremor Detector Started ===

RMS=0.0132g  --> NORMAL

RMS=0.0471g  --> MILD

If nothing appears:

- Check that USART2 is configured for PA2/PA3 in CubeMX and MX_USART2_UART_Init() is generated.

- Ensure debug probe and ST-LINK VCP drivers are installed (ST site).

- Some Nucleo boards need the VCP enabled; check board docs.

# 9) How to test the sensor right away

- Keep the MPU steady on desk: RMS should show NORMAL.

- Tap or shake lightly: it may show MILD or MODERATE.

- Hold the sensor and move normally (voluntary movement): might show spurious levels — we’ll filter/label later.

# 10) Collecting labeled data for better classification

You want very specific classification → you need labeled data.

Simple data collection approach:

- Modify the code to print raw accel values and timestamps to UART (CSV like ts,ax,ay,az,label).

- For each recording session, manually label periods as NORMAL, TREMOR_MILD, TREMOR_SEVERE.

- Save data on PC and train a classifier (I can help write the Python script to train).

- Export trained model or convert into thresholds or a small decision tree for MCU.

I can provide the code to log raw samples and a Python notebook if you want.

# 11) Common problems & fixes

- SPI read returns garbage: Check CS pin wired to PA4, set PA4 high in code before init, check SPI prescaler (too fast). In CubeMX set SPI prescaler slower (e.g., 16 or 32) while testing.

- Compile error: undefined hspi1 or huart2: Make sure CubeMX created spi.c and usart.c and that you didn’t name the handles differently.

- No serial output: Ensure printf retargeting _write() uses huart2. Make sure ST-LINK VCP installed.

- Device doesn’t wake: Try writing REG_PWR_MGMT_1 with 0x80 reset, delay, then 0x00 to wake.

ML MODEL TRAINING

I’ll assume:

- Your working PC has Windows (commands for macOS/Linux are similar; say if you need them).

- STM32CubeIDE is installed and set up.

- You can build and flash code to Nucleo-F401RE already (you do, since sensor worked).

- You’ll use UART (USART2) for serial logging at 115200 baud.

- You will use X-CUBE-AI inside STM32CubeIDE to import the .tflite model and generate C code.

If you want another deployment path (TFLite Micro manual), I can give that after but X-CUBE-AI is easiest for beginners.

# For ML Training:

# STEP 0 — Prep (installations, folders)

- Create a project folder on your PC:

  - C:\tremor_project\

  - Subfolders:

    - C:\tremor_project\mcu_code\

    - C:\tremor_project\data\raw\

    - C:\tremor_project\data\windows\

    - C:\tremor_project\python\

  - C:\tremor_project\model\

  Install Python 3.10+ (from python.org) if not present.

  Open a command prompt (Win+R → cmd) and create a venv:

cd C:\tremor_project

python -m venv venv

venv\Scripts\activate

pip install --upgrade pip

pip install numpy pandas scipy matplotlib scikit-learn tensorflow pyserial

Make sure STM32CubeIDE installed and you have X-CUBE-AI plugin:

- Open STM32CubeIDE.

- Help → Install New Software → search X-CUBE-AI and install if missing OR download X-CUBE-AI from ST and install.
(If you don’t have it, you can still use TFLite Micro, but X-CUBE-AI is simpler.)

# STEP 1 — Modify MCU to stream raw sensor data

You already have mpu9255 driver. We’ll add UART streaming in CSV.

## MCU changes (exact code edits)

- Open your STM32CubeIDE project (the one you used before).

- In main.c (or a new file), make sure huart2 is declared (from MX_USART2_UART_Init). Add/keep _write you had to allow printf.

- Replace the sampling loop temporarily with this streaming code (copy-paste):

// Add at top

#include <stdio.h>

#include "mpu9255.h"

extern UART_HandleTypeDef huart2;

// keep your _write() already present

int _write(int file, char *ptr, int len) {

HAL_UART_Transmit(&huart2, (uint8_t*)ptr, len, HAL_MAX_DELAY);

return len;

}

void stream_mode(void) {

mpu9255_init();

printf("ax,ay,az\r\n");

while (1) {

int16_t ax, ay, az;

mpu9255_read_accel_raw(&ax, &ay, &az);

// print raw ints

printf("%d,%d,%d\r\n", ax, ay, az);

HAL_Delay(5); // 200 Hz -> 5 ms

}

}

- In main() after MX init functions, call stream_mode(); (instead of your tremor loop).

- Build (hammer icon) → Fix any missing includes (stdio.h added). Flash to board (run).

## Serial capture on PC (click-by-click)

- Connect Nucleo via USB to PC.

- Open Tera Term or PuTTY (or STM32CubeIDE Serial Terminal).

  - Serial port: your Nucleo COM port (check Device Manager).

  - Baud: 115200, Data: 8, Parity: None, Stop: 1, Flow control: None.

- Start terminal → you should see header ax,ay,az.

## Save CSV files for each class (exact procedure)

- For each label (NORMAL, MILD, MODERATE, SEVERE) do:

  - Prepare the subject to perform that motion.

  - In terminal, click File → Log (TeraTerm) or Session → Logging.

  - Save as: C:\tremor_project\data\raw\normal_01.csv (use unique names per session).

  - Perform the motion for 20 seconds.

  - Stop logging and repeat for multiple sessions (10+ per class at start).

Aim for at least: 200 windows per class (1-second windows with overlap later). That means ~200s total per class, but you can start smaller (50 windows/class) to iterate.

# STEP 2 — Convert raw CSVs to labeled windows

Now we convert stream-of-rows into fixed 1-second windows of 200 samples and label each window.

Create C:\tremor_project\python\make_windows.py with the code below. Then run it.

Run:

venv\Scripts\activate

python C:\tremor_project\python\make_windows.py

You’ll get windows.npz in data\windows.

# STEP 3 — Feature extraction OR raw-input format

You can choose 2 paths:

- Feature-based model (smaller, easier for MCU): extract ~12 features per window.

- Raw-signal 1D-CNN (more accurate often, but bigger model).

I’ll give both; start with feature-based (recommended).

## Feature extraction script (create features.npy)

Create C:\tremor_project\python\features.py:

Run:

python C:\tremor_project\python\features.py

Now you have features.npz containing X (n_samples, n_feats) and y.

# STEP 4 — Train a small ML model (Keras) and export TFLite

We’ll train a tiny dense model on features. Then convert & quantize.

Create C:\tremor_project\python\train_model.py

Run:

python C:\tremor_project\python\train_model.py

Confirm keras_model.h5 saved.

## Convert to TFLite + int8 quantization

Create C:\tremor_project\python\convert_tflite.py:

Run:

python C:\tremor_project\python\convert_tflite.py

You now have tremor_model_int8.tflite in C:\tremor_project\model

# STEP 5 — Import .tflite into STM32CubeIDE using X-CUBE-AI

Note: X-CUBE-AI accepts Keras .h5 as well; I’ll show .tflite pathway for clarity. If X-CUBE-AI wants .h5, use keras_model.h5 instead.

## Option A — Using X-CUBE-AI (GUI)

- Open STM32CubeIDE and your project (C:\tremor_project\mcu_code project).

- Right click on project → Properties → C/C++ Build → Settings → ensure MCU GCC Linker and memory settings are default. Close.

- Right click project → New → STM32CubeMX Configuration (or open existing MX config).

- In the CubeMX perspective, go to AI Core or AI (depends on plugin version):

  - There will be an AI Modeling section or a tab X-CUBE-AI.

- Click Import Model → choose C:\tremor_project\model\tremor_model_int8.tflite (or .h5 if required).

- Use default settings initially:

  - Target: STM32F401RE

  - Optimize for speed/memory: choose Memory

  - Input type: int8 (since quantized)

Click Generate. X-CUBE-AI will create code files: ai_model.c, ai_model.h, ai_datatypes.h, the model   weights as arrays, and example wrapper functions.

  Click Generate Code then Open Project.

# STEP 6 — Integrate AI inference into your firmware (detailed edits)

After code generation, you’ll have AI API functions. Typical workflow:

- In your project, open app_ai.c or where X-CUBE-AI instructs to call model.

- Add includes:

#include "ai_model.h"

#include "ai_datatypes_defines.h"

- Allocate input and output buffers (match model input shape). X-CUBE-AI usually provides helper APIs. Example pseudo:

// before main loop

ai_handle network = NULL;

ai_network_params params;

ai_buffer ai_input[AI_NETWORK_IN_NUM];

ai_buffer ai_output[AI_NETWORK_OUT_NUM];

// initialize network

ai_err err = ai_network_create(&network, AI_NETWORK_DATA_CONFIG);

err = ai_network_init(network, &params);

- Preprocess on MCU:

  - You extracted features on PC. Do the same on MCU: compute the same 13 features for each 1-second window.

  - Apply the same normalization: (x - mean)/std. The mean and std you saved earlier (norm_stats.npz) — convert them to C arrays and add to project.